The solution should include the following networking elements:

- CCTV System (IP cam, NVRs, VMS, etc)

- Passive Parts

- Switches (access , distribution, core)

- Every three floors are connected to a one switch and one cabinet with all accessories.

- Excluded from the previous point the (1st & GF & B) Floors.

- Every switch installed on the (1st & GF & B) floor must be separately connected to the control room.

- The Parking and The side building must be connected by Fiber uplink

- All Uplinks connected directly to the control room must be redundant

- Specifications

The proposed solution should comply with the below technical requirement. The requirement should be answered with Compliant (C), Partial Compliant (PC) or Not Compliant (NC).

Please note that the “Description of Feature” column should be filled out to describe how the specific requirement is met by the proposed solution.

Service & Support must be 3 years (parts and Labour) onsite warranty for all components.

- Switches1 (Qty = 2)

Min. Specs

| A.Distributed Switches Specifications |  | Comply |  |
| --- | --- | --- | --- |
|  |  | Yes | No |
| 1. | Resiliency & High Availability |  |  |
| 1.1 | o Stackable minimum 24Gbps stacking throughput each |  |  |
| 1.2 | o Stackable up to 8 units in a stack |  |  |
| 2. | Performance |  |  |
| 2.1 | o 24 10/100/1000 with 4 SFP+ interface accepting 1/10G transceivers. |  |  |
| 2.2 | o Minimum forwarding ratre of 90 Mpps |  |  |
| 2.3 | o The Switches should support PoE+ 802.3at |  |  |
| 2.4 | o Supports minimum power budget of 720W per switch |  |  |
| 3. | Features & Standards |  |  |
| 3.1 | o Minimum 16,000 MAC address supported |  |  |
| 3.2 | o IEEE 802.1 Q VLAN encapsulation. |  |  |
| 3.3 | o Link aggregation 802.1ad and LACP. |  |  |
| 3.4 | o Support for Jumbo Frames |  |  |
| 3.5 | o IGMP v1/v2/v3 |  |  |
| 3.6 | o L2/L3/L4 Access Control Lists (ACL) on all ports |  |  |
| 3.7 | o Support for eight egress QoS queues per port |  |  |
| 3.8 | o Per port shaping and policy. |  |  |
| 3.9 | o LLDP for network topology discovery |  |  |
| 3.10 | o Radius/TACACs+ Authentication Support |  |  |
| 3.11 | o SNMP v1, 2, 3 |  |  |
| 3.12 | o GUI based configuration and Monitoring,CLI |  |  |

# Switches2 (Qty = 17)

| Access Switches Specifications |  | Comply |  |
| --- | --- | --- | --- |
|  |  | Yes | No |
| 1. | Resiliency & High Availability |  |  |
| 1.1 | o Stackable minimum 24Gbps stacking throughput each |  |  |
| 1.2 | o Stackable up to 7 units in a stack |  |  |
| 2. | Performance |  |  |
| 2.1 | o 24 10/100/1000 PoE+ ports with 4 SFP+ interface accepting 1/10G transceivers. |  |  |
| 2.2 | o Minimum forwarding ratre of 90 Mpps |  |  |
| 2.3 | o The Switches should support PoE+ 802.3at |  |  |
| 2.4 | o Supports minimum power budget of 720W per switch |  |  |
| 3. | Features & Standards |  |  |
| 3.1 | o Minimum 15,000 MAC address supported |  |  |
| 3.2 | o IEEE 802.1 Q VLAN encapsulation. |  |  |
| 3.3 | o Support for Jumbo Frames |  |  |
| 3.4 | o Port security to limit allowed mac addresses. |  |  |
| 3.5 | o Support for eight egress QoS queues per port |  |  |
| 3.6 | o Per port shaping and policing. |  |  |
| 3.7 | o LLDP for network topology discovery |  |  |
| 3.8 | o Radius/TACACs+ Authentication Support |  |  |
| 3.9 | o SNMP v1, 2, 3 |  |  |
| 3.10 | o GUI based configuration and Monitoring,CLI |  |  |

| Switch Specifications Within the floor if needed |  | Comply |  |
| --- | --- | --- | --- |
|  |  | Yes | No |
| 1. | Resiliency & High Availability |  |  |
| 1.1 | o Stackable up to 4 units in a stack |  |  |
| 2. | Performance |  |  |
| 2.1 | o 12 10/100/1000 PoE+ ports with 2 SFP+ interface accepting 1/10G transceivers. |  |  |
| 2.2 | o The Switches should support switching Capacity at least 64Gbps. |  |  |
| 2.3 | o Minimum forwarding rate of 47Mpps |  |  |
| 2.4 | o Switches should support PoE+ 802.3at |  |  |
| 2.5 | o Supports minimum power budget of 123W per switch |  |  |
| 3. | Features & Standards |  |  |
| 3.1 | o Minimum 15,000 MAC address supported |  |  |
| 3.2 | o IEEE 802.1 Q VLAN. |  |  |
| 3.3 | o Link aggregation 803.1ad and LACP. |  |  |
| 3.4 | o Support for Jumbo Frames |  |  |
| 3.5 | o IGMP |  |  |
| 3.6 | o Classification type of service (ToS) or DSCP. |  |  |
| 3.7 | o LLDP for network topology discovery |  |  |
| 3.8 | o Radius/TACACs+ Authentication Support |  |  |
| 3.9 | o SNMP v1, 2, 3 |  |  |
| 3.10 | o GUI based configuration and Monitoring and support SSH and CLI |  |  |
| 3.11 | o Rack mounted kit should be included |  |  |

# Switches3 (Qty = 4)

CCTV System

- 4 MP indoor IP dome Camera (Qty=120(

|  | Feature | Minimum Specification | Comply |  |
| --- | --- | --- | --- | --- |
|  |  |  | Yes | No |
| 1 | Image sensor | progressive scan CMOS |  |  |
| 2 | Lens | 2.8mm lens |  |  |
| 3 | Compression | H.265,H.264, three streams |  |  |
| 4 | Resolution | 4MP |  |  |
| 5 | FPS | 25fps @ 2MP |  |  |
| 6 | Day and night | Yes (IR cut filter) |  |  |
| 7 | Minimum illumination | 0.05 lux (color)/ 0 lux (IR on). |  |  |
| 8 | IR range | Min 40 meters |  |  |
| 9 | Protocols | TCP/IP, ICMP, HTTP, HTTPS, FTP, DHCP, DNS, DDNS, RTP, RTSP, RTCP, PPPoE, NTP, UPnP, SMTP, SNMP, IGMP, QoS, IPv6 IPv4, UDP, Bonjour |  |  |
| 10 | Image Processing | Wide Dynamic Range 120dB, BLC, HLC 3D Digital Noise Reduction |  |  |
| 11 | Interface | 10/100 Ethernet |  |  |
| 12 | Local Storage | Support Built-in Micro SD card Slot up to 128 GB |  |  |
| 13 | Power | PoE |  |  |
| 14 | Approvals | CE compliant (certificate must be attached) |  |  |
| 15 | Compatibility | ONVIF |  |  |
| 16 | Protection levels | IK10, IP67 |  |  |
| 17 | Alarm Trigger | Motion detection, video tampering, network disconnected, IP address conflict, illegal login, HDD error, HDD full |  |  |

- 4 MP Outdoor IP Bullet Camera	 (Qty.: 80)

|  | Feature | Minimum Specification | Comply |  |
| --- | --- | --- | --- | --- |
|  |  |  | Yes | No |
| 1 | Image sensor | progressive scan CMOS |  |  |
| 2 | Lens | Vari-Focal Motorized lens2.7~13.5 mm, auto iris |  |  |
| 3 | Compression | H.265,H.264, three streams |  |  |
| 4 | Resolution | 4 MP |  |  |
| 5 | FPS | 30fps @ 4MP |  |  |
| 6 | Day and night | Yes (IR cut filter) |  |  |
| 7 | Minimum illumination | 0.05 lux (color)/ 0 lux (IR on). |  |  |
| 8 | IR range | Min 60 meters |  |  |
| 9 | Protocols | TCP, HTTP, DHCP, DNS, NTP, RTP, 802.1X |  |  |
| 10 | Image Processing | Wide Dynamic Range 120 dB, 3D Digital Noise Reduction, HLC, region of interest (ROI) |  |  |
| 11 | Smart features | Motion detection, , line crossing, video tampering, region (entrance/exit) detection, Unattended baggage detection, face detection, scene change detection |  |  |
| 12 | Interface | 10/100 Ethernet |  |  |
| 13 | Local Storage | Built-in Micro SD card Slot up to 128 GB |  |  |
| 14 | Power | PoE |  |  |
| 15 | Approvals | CE compliant (certificate must be attached) |  |  |
| 16 | Compatibility | ONVIF |  |  |
| 17 | Interface | Video output, alarm input/output |  |  |
| 18 | Weather Proof | IP67 |  |  |
| 19 | Vandal proof | IK10 |  |  |

- PTZ Camera (Qty=4)

|  | Feature | Minimum Specification | Comply |  |
| --- | --- | --- | --- | --- |
|  |  |  | Yes | No |
| 1 | Image sensor | 1/3 progressive scan CMOS |  |  |
| 2 | Optical zoom | 40 X optical zoom, 12X digital zoom |  |  |
| 3 | Compression | H.265, H.264, at least 3 streams |  |  |
| 4 | Resolution | 2 MP |  |  |
| 5 | FPS | 25fps @ 2MP |  |  |
| 6 | PTZ Range | 360° endless, -10°-90° auto flip |  |  |
| 7 | Day and night | Yes (IR cut filter) |  |  |
| 8 | Minimum illumination | 0.005 lux (color)/ 0 lux (IR on). |  |  |
| 9 | IR range | Up to 150 meters |  |  |
| 10 | Protocols | TCP, HTTP, DHCP, DNS, NTP, RTP |  |  |
| 11 | Image Processing | Digital Wide Dynamic Range ,3D Digital Noise Reduction, Defog, HLC, BLC, Regional exposure, regional focus |  |  |
| 12 | Smart features | Motion detection, video tampering, intrusion detection , line crossing, region (entrance/exit) detection |  |  |
| 13 | Interface | 10/100 Ethernet |  |  |
| 14 | Power | PoE |  |  |
| 15 | Approvals | CE compliant (certificate must be attached) |  |  |
| 16 | Compatibility | ONVIF |  |  |
| 17 | Weather Proof | IP66 |  |  |

# Passivepart

|  | Features | Comply |  |
| --- | --- | --- | --- |
|  |  | Yes | NO |
| 1. | UTP |  |  |
| 1.1 | Cat6 UTP Cable : |  |  |
| 1.2 | Cat.6, U/UTP, AWG23 cable |  |  |
| 1.3 | frequencies of up to 450 MHz |  |  |
| 1.4 | ISO/IEC 11801 ed. 2.2; IEC 61156-5 2nd ed.; EN 50173-1; EN 50288-6-1;EIA/TIA 568-C.2 |  |  |
| 2. | Cat.6 connection: |  |  |
| 2.1 | RJ45 Cat.6 connection module Tool free |  |  |
| 2.2 | Dust cover |  |  |
| 2.3 | IEC 60603-7: Electrical Characteristics of the Telecommunication Outlets ISO/IEC 11801 |  |  |
| 2.4 | EN 50173-1 & TIA 568-A / TIA 568-B |  |  |
| 2.5 | Material: plastic: PC, UL |  |  |
| 3. | Patch Cords Cat.6: |  |  |
| 3.1 | Comply with the Category 6 - IEC 60603-7-4 / IEC 60603-7-5 |  |  |
| 3.2 | Tested of IEC 61935-2 |  |  |
| 3.3 | Available with Security system –Option |  |  |
| 3.4 | Lengths 0.5 or 1.0 Meter |  |  |
| 4. | 24-Port Patch Panel |  |  |
| 4.1 | Modular design |  |  |
| 4.2 | Snap-in Connecter |  |  |
| 4.3 | 1U |  |  |
| 5. | Faceplate: ( QTY 200 ) |  |  |
| 5.1 | 86x86 mm |  |  |
| 5.2 | Blind cover |  |  |
| 5.3 | Halogen-free |  |  |
| 5.4 | Capacity 1 or 2 ports |  |  |
| 5.5 | suitable for Security system –Option |  |  |
| 6. | Fiber cabling: |  |  |
| 6.1 | Fiber cabling must be MM and supporting 10G or above for all items ( Cable , Patch panels, patch cords and connectivity ) |  |  |
| 7. | Cabinets: 6U or 9U |  |  |
| 7.1 | Standards: |  |  |
| 7.1.1 | ISO90001:2000; IEC 60917 and IEC 60297 |  |  |
| 7.1.2 | Black 19” wall mount cabinet with glass front door |  |  |
| 7.2 | Load Capacity: 60-80KG |  |  |
| 7.3 | 2 ceiling fans |  |  |
| 7.4 | 1x6-way MK PDU |  |  |
| 7.5 | Side doors: lockable, removable complete with locks |  |  |
| 8. | Cabinets : 42U Server cabinet |  |  |
| 8.1 | Standards: |  |  |
| 8.1.1 | Comply with ANSI/EIA |  |  |
| 8.1.2 | RS-310-D |  |  |
| 8.1.3 | IE660297-2 |  |  |
| 8.2 | Material : |  |  |
| 8.2.1 | SPCC cold Rolled steel |  |  |
| 8.2.2 | 19" rails:2.0 mm |  |  |
| 8.2.3 | Casters tray : 2.0 mm |  |  |
| 8.3 | Load Capacity: 1300KG |  |  |
| 9. | Notes |  |  |
| 9.1 | All Cat6 / fiber cables, accessories and Cabinets should be same brand. |  |  |
| 9.2 | Brand accepted USA and Europe, |  |  |
| 9.3 | Installation team must be cabling certified. |  |  |

- NVR 32 CH (Qty=7)

| Minimum Standard | Comply |  |
| --- | --- | --- |
|  | Yes | No |
| 32 Channel NVR |  |  |
| Live view; recording and playback simultaneously |  |  |
| Supported encoding (H.265+,H.265,H.264, MJPEG) |  |  |
| Multiple USB ports including USB 3.0 |  |  |
| Multi user roles and authentication levels |  |  |
| Can manage all cameras mentioned above |  |  |
| 8sata interfaces per NVR with HDD up to 10TB |  |  |
| HDMI video output:2 outputs with one at least up to 4K resolution, |  |  |
| Support RAID 0/1/5/10 |  |  |
| Bidder must provide all needed licenses |  |  |
| Must support 3rd party cameras |  |  |
| Hot spare N+1 configuration available |  |  |
| Recording resolution: up to 12MP |  |  |
| Must support video content analysis & point of sale |  |  |
| Network: 2 Rj-45 connections with at least 300 Mbps |  |  |

- Screens (Qty 8):

- 50 inch screen.

- 3 input HDMI.

- USB port.

- VGA PORT.

- WALL MOUNT INCLUDED

- VMS Min Specs

| Feature |  |  | Performance Specification | Comply |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Yes | No |  |
| 1 | General | Cameras | Support min. 1500 cam |  |  |  |
|  |  | Managed Device IP Addresses *Including Encoding Devices, Access Control Devices, Elevator Control Devices, Security Control Devices, and Remote Sites | 2000 |  |  |  |
|  |  | Video Intercom Devices | 1,000 |  |  |  |
|  |  | Alarm Inputs (Including Alarm Inputs of Security Control Devices) | Support |  |  |  |
|  |  | Alarm Outputs | Support |  |  |  |
|  |  | Alarm Inputs of Security Control Devices | Support |  |  |  |
|  |  | Recording Servers | 64 |  |  |  |
|  |  | Streaming Servers | 64 |  |  |  |
|  |  | Security Audit Server | 8 |  |  |  |
|  |  | People Counting Cameras | 300 |  |  |  |
| 2 | Recording | Recording Schedule | 10000 |  |  |  |
|  |  | Recording Schedule Template | 200 |  |  |  |
| 3 | Event & Alarm | Event and Alarm without picture | Support |  |  |  |
|  |  | Storage of Events or Alarms | Support |  |  |  |
|  |  | Events or Alarms Sent to Clients *The clients include Control Clients and Mobile Clients. | 100/s 100 Clients/s |  |  |  |
| 4 | Picture | Picture Storage *Including event/alarm pictures, face pictures, and vehicle pictures. | 20/s (Stored in SYS Server) 100/s (Stored in Recording Server) |  |  |  |
| 5 | Reports | Regular Report Rules | 100 |  |  |  |
|  |  | Event or Alarm Rules in One Event/Alarm Report Rule | 32 |  |  |  |
|  |  | Records in One Sent Report | 10,000 or 10 MB |  |  |  |
| 6 | Data Storage | Data Retention Period | Stored for 3 Years |  |  |  |
|  |  | People Counting | 5 million |  |  |  |
|  |  | Heat Map | Support |  |  |  |
|  |  | Events | 60 million |  |  |  |
|  |  | Alarms | 60 million |  |  |  |
|  |  | Access Records | 1.4 billion |  |  |  |
|  |  | Visitor Records | 10 million |  |  |  |
|  |  | Operation Logs | 5 million |  |  |  |
|  |  | Service Information Logs | 5 million |  |  |  |
|  |  | Service Error Logs | 5 million |  |  |  |
|  |  | Recording Tags | 60 million |  |  |  |
| 7 | Users and Roles | Concurrent Accesses via Web Clients, Control Clients, and OpenAPI Clients | 100 |  |  |  |
|  |  | Concurrent Accesses via Mobile Clients and OpenAPI Clients | 100 |  |  |  |
|  |  | Users | 2500 |  |  |  |
|  |  | Roles | 3000 |  |  |  |
| 8 | Smart Wall | Decoding Devices | 32 |  |  |  |
|  |  | Smart Walls | 32 |  |  |  |
|  |  | Views | 1000 |  |  |  |
|  |  | View Group | 100 |  |  |  |
|  |  | Views in One View Group | 10 |  |  |  |
|  |  | Cameras in One View | 150 |  |  |  |
|  |  | Views Auto-Switched Simultaneously | 32 |  |  |  |

CCTV Solutions

The required for project is about the capability to record systems, store the video from IP-cam to NVRs.

Locally Recording

The resolution recording for indoor/outdoor IP-cam it will be on

2M resolution (1920x1080)FHD

Frame bit: 15fps continues

Regarding recording/storing   180 days for indoor/outdoor IP-camis needed for the project. In an effort to provide efficient performance of a centralized management system, growth, and a step toward fully functional surveillance system, the IT department has identified the following needs:

- Type of deployment SHOULD BE centrally managed (The centralized video management system (VMS), video management server, and storage (storage area networks) are an integral point of a video surveillance system. Each component must be designed and configured correctly by certified partner.

- Licensing SHOULD BE in details (price, used for, Qty, renewal ,upgrade ,level support)

- The VMS SHOULD Support Dashboard for monitoring devices and servers

- The VMS SHOULD Support maintenance with remote access

- The VMS SHOULD Support Web based SDK (None plug-in SDK)

- The system should be capable of managing large bandwidth requirements from live video and the storage to retrieve the video files for playback and management of stored video.

- The video management system must be able to support multiple clients logged into the system concurrently to view live video.

- The video management system must be able to support dual streaming of video for live view and recorded video. It must be able to support compression of individual streams for optimized resolution and frame rate

- built-in video motion / face detection, and privacy masking

- Provide support for native 64-bit operating systems

- Instant one-click playback for easy viewing of exported video evidence

- Ability use evidence to generate a printed  (report, a JPEG image, an AVI film) or in exported format with built in player feature

- Ability to create report depending on VMS features available

- System login using client user name and password

- Secure connection through HTTPS