| Category | Specifications |
| --- | --- |
| Structure & Materials | Stainless steel / powder-coated aluminum frame (anti-corrosion) Load capacity: up to 200 kg |
| Water Tank / Chamber | Capacity: 800–1800 liters Adjustable water depth Automatic filling & drainage system Integrated filtration & disinfection (UV or Ozone or Chlorine) Overflow and level control sensors |
| Treadmill Belt System | anti-slip running surface Speed range: to be mentioned Incline adjustment Bidirectional movement (forward/reverse) |
| Control & Interface | Waterproof touch screen panel Adjustable functions: speed, water level, temperature (15–40 °C), Pre-programmed training & rehab modes Emergency stop buttons (internal & external) |
| Safety & Ergonomics | Low entry step / access ramp Internal support rails & anti-slip handles |
| Power & Utilities | Power supply: 220–240 V, 50/60 Hz (single or three-phase( Integrated water heating system (electric heater or heat pump) Drainage: connection to facility plumbing |
| Dimensions (Typical) | External: L( 2200–3000) mm, W (1000–1300) mm, H (1200–1600) mm Internal: suitable for users up to (190–200) cm height Dry weight:( 500–1200) kg |