# Lot 1: Tape Library   مكتبة أشرطة نسخ احتياطي

QTY 1

| Features | Required Specification |  | Comply (Y/N) |
| --- | --- | --- | --- |
| Base | RACK MOUNT with all needed accessories to install in a rack cabinet with all needed cables |  |  |
| Tape drive | Two FC LTO9 Tape Drive |  |  |
| Power | Redundant Power Supplies |  |  |
| Slots | Installed with a minimum of 30 slots |  |  |
| Interface | Min 8Gb/s FC |  |  |
| Cartridges | Installed with 20 LTO-9 cartridges with label 2 cleaning cartridges |  |  |
| Management | TCP/IP management port, Web management |  |  |
| Barcode | must include barcode readers |  |  |
| LCD Display | Must has an LCD display and indicators for Power, Drives and Activity, error status and message information |  |  |
| Warranty | 5 Years warranty |  |  |
| Module Name | Description | Qty |  |
| Base | TS4300 tape base module | 1 |  |
| Tape drive | LTO9 HH fiber channel drive | 2 |  |
| Indicator | Standard S&H indicator | 1 |  |
| Rack kit | Rack mount kit\ | 1 |  |
| Power Supply | Dual Power Supply | 1 |  |
| Tape cartridges | LTO9 tape cartridges | 20 |  |
| Cleaning cartridges | Cleaning tape | 2 |  |
| Warranty | 5 Years mother company support | 1 |  |

# Lot 2: VEEAM backup Software

| Product | Qty |
| --- | --- |
| Veeam Data Platform Advanced Universal Subscription License. Includes Enterprise Plus Edition features. 10 instance pack. 5 Years Subscription Upfront Billing & Production (24/7) Support. | 1 |

# Lot 3: Firewalls

QTY:2

| # | Technical Specification | Full Compliance (Yes/No) |
| --- | --- | --- |
| 1 | Preferred vendors: Fortinet, Cisco and Paloalto |  |
| 2 | Solution should be an integrated Next Gen Firewall platform which includes firewall, application control, IPS, URL Filtering and Advanced Persistent threat Prevention capabilities in a single appliance from day one for 5 years. |  |
| 3 | Feature consistency in hardware and virtualized form factors. The same features available in hardware form factor is also available in a virtualized form factor to help organizations support their virtualization initiatives. |  |
| 4 | The appliance should support at least 8x 5/2.5 Gigabit ports from Day one. Must support at least 10x 1G SFP ports include 4x 1G short range (multi-Mode) transceiver and 4x 10G SFP+ ports include 4x 10G short range (multi-Mode) transceiver |  |
| 5 | Proposed appliance should have dual power supply. |  |
| 6 | Firewall Throughput must be more than 25 Gbps production / real world throughput considering environment with real-world traffic blend, multiple Security features, a typical rule-base size, NAT and logging enabled. Data need to be substantiated and available in public domain (links to be provided). |  |
| 7 | Proposed IPS throughput with TCP and UDP traffic blend, with customized IPS Policy, scanning all part of sessions in both directions must |  |
| 8 | Firewall should support 3DES/AES IPsec VPN throughput of at least 15 Gbps |  |
| 9 | Firewall should support at least 12 million concurrent sessions. |  |
| 10 | Firewall should support minimum 600,000 new connections per second. |  |
| 11 | Firewall should support minimum 12 Gbps SSL Inspection Throughput |  |
| 12 | Network Security Firewall should support "Stateful" policy inspection technology. It should also have application intelligence for commonly used TCP/IP protocols like telnet, ftp etc. |  |
| 13 | The appliance should support Active-Active or Active-Standby Deployment |  |
| 14 | Appliance should have granular visibility with respect to user and group policy. |  |
| 15 | The Firewall should support authentication protocols like Active Directory, LDAP and Radius |  |
| 16 | The IPS should be constantly updated with new defenses against emerging threats. |  |
| 17 | IPS Engine should support Vulnerability and Exploit signatures, Protocol validation, Anomaly detection, Behavior-based detection, multi-element correlation. |  |
| 18 | IPS should have the functionality of Geo Protection to Block the traffic country wise incoming as well as outgoing. |  |
| 19 | IPS should be able to detect and prevent embedded threats with in SSL traffic. |  |
| 20 | Application control must Identify applications, its different categories, URLs, HTTPS inspection, Malware content sites, IP and/or user-based policies. |  |
| 21 | Inspect content to protect networks. Protect the network against known and unknown threats including vulnerability exploits and malware embedded in all types of traffic. |  |
| 22 | Solution must have a URL categorization that exceeds 1 million URLs filtering database. Should have pre-defined over 90 categories. The solution should have the capabilities to block, permit, allow & log, protocols other than HTTP, HTTPs, FTP. Bidder to confirm URL database. |  |
| 23 | The solution should also have the scalability to scan & secure SSL encrypted traffic passing through gateway. Should perform inspection to detect & block malicious content downloaded through SSL. |  |
| 24 | Identify malware signatures and malicious web sites. |  |
| 25 | Granularly define exceptions for SSL inspection to protect user privacy and comply with corporate policy. |  |
| 26 | 5 Years Mother Company warranty and devices should be under KAUH name and bidder should provide contract number with vender that contains all devices Serial’s Covered (Include Hardware and Software) |  |
| Security Management Specifications |  |  |
| 27 | reporting, logging and analyzer solution has to be dedicated appliance or VM |  |
| 28 | Central management and reporting solution should support log rate at least 4000 logs per second and 25G per Day. |  |
| 29 | Management (Management, reporting, analysis) System Support for role-based administration of firewall |  |
| 30 | Solution should support analysis of traffic pattern using graphs and charts |  |