المواصفات المطلوبة للمناقصة رقم (ل ش 344/2025) الخاص بشراء

Barcode Reader Scanner

General Specifications:

- Scanner Type Technology: General-duty cordless barcode Reader scanner.

- Decode Capabilities: Area Imager (supports both ID and 2D barcodes such as QR, DataMatrix (PDF417)

- Wireless Technology: Bluetooth v4.2

- Wireless Range: Up to 30 meters (98 feet) with charger/base.

- Scan Pattern: Area Image (1040 x 720-pixel array).

- Motion Tolerance: 70 cm/s for 13 mil UPC.

- Scan Angle: Horizontal: 39.2° nominal; Vertical: 27.4° nominal.

- User Indicators: Good Decode LEDs, Rear View LEDs, and an adjustable beeper.

- Durability: Withstands 30 drops from 1.8 meters (6 ft) to concrete and 1,000 tumbles at 0.5 meters (1.6 ft).

- Environmental Sealing: IP42-rated for protection against dust and water.

- Battery Life: Up to 14 hours of operation.

- Charging Time: Approximately 4.5 hours.

- Durability: Built to withstand demanding point-of-sale environments.

- Optimized for: Digital coupons, wallet apps, and on-screen barcodes from smartphones

- Connectivity: Bluetooth and USB (Plug-and-Play) options available.

- Warranty option 1: 3-year, Warranty option 2: 5-year limited warranty

- ملاحظة هامة: احضار عينة للتجريب